# Government Notice  No.  93 published on  9/3/2018

THE URBAN PLANNING ACT

(Cap. 355)

____________

REGULATIONS

____________

(Made under section 77 (1) (b))

____________

# THE URBAN PLANNING (PLANNING SPACE STANDARDS) REGULATIONS, 2018

TABLE OF CONTENTS

## PART I

## PRELIMINARY PROVISIONS

- Citation

- Application

- Interpretation

## PART II

## URBAN PLANNING SPACE STANDARDS

- Functional space

## PART III

## REVOCATION

- Revocation

_______

SCHEDULE

________

THE URBAN PLANNING ACT

(CAP. 355)

____________

REGULATIONS

____________

(Made under section 77 (1) (b))

____________

THE URBAN PLANNING (PLANNING SPACE STANDARDS) REGULATION, 2018

| PART I PRELIMINARY PROVISIONS |  |
| --- | --- |
| Citation | 1. These Regulations may be cited as the Urban Planning (Planning and Space Standards) Regulations, 2018. |
| Application | 2. These regulations shall apply to all planning areas declared by the Minister under section 8 of the Act. |
| Interpretation | 3. In these Regulations unless the context require otherwise:- |
|  | "Urban Planning Space Standards" include standards for residential areas, unplanned settlements, building height, building lines and setbacks, floor are, plot coverage and plot ratio, health facilities, education facilities, recreation facilities, beach facilities, golf course, passive and active recreation, public facilities by planning levels, public facilities by population size, parking and road width and agricultural show grounds. |
| PART II URBAN PLANNING SPACE STANDARDS |  |
| Functional space | 4.-(1) Adequate and functional space shall be allocated in accordance with the Urban Planning Space Standards prescribed in the Schedule to these Regulations. |
|  | (2) The space standards used shall be applied to create compact towns. |
|  | (3) The space standards used shall be applied to create compact towns. |
|  | (4) Space provided for show grounds and urban agriculture shall be distributed in such a manner that they can easily accommodate built environment in the future with mere change of land use. |
|  | (5) Specific standards shall be used to control minimum spaces between buildings. |
| PART III REVOCATION |  |
| Revocation GN. Nos. 157 of 1997 and 395 of 2011 respectively | 5. The Town and Country (Town Planning Space Standards) Regulations of 1997 and of 2011 are hereby repealed. |

_________

SCHEDULE

__________

URBAN PLANNING SPACE STANDARDS

(Made under regulation 4(1))

___________

- General Standards

- Standards for Residential Areas - Detached house and Maisonettes

|  | Type | Plot Size in per-urban area | Max. No. of household | Max. No. of Buildings | Max. Plot coverage % | Max. Plot ratio | Max. No. of storey | Minimum Setbacks in Metres |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | front | sides | rear |
| i | Special area and unplanned settlements (Special case) | 90 - 300 | 1 | 1 | 80 | 2.5 | 4-6 | 5 | 1.5 | 3 |
| ii | High density | 301 – 600m2 | 1 | 2 | 60 | 2.0 | 4-6 | 5 | 1.5 | 3 |
| iii | Medium density | 601 – 800m2 | 1 | 2 | 55 | 1.5 | 4-6 | 7 | 2 | 4 |
| iv | Low density | 801 – 1200m2 | 1 | 2 | 50 | 1.0 | 4-6 | 10 | 2.5 | 5 |
| v | Super Low density | 1201 – 2000m2 | 1 | 2 | 45 | 0.7 | 4-6 | 12 | 3 | 5.5 |

- Specific  Standards

- Standards for Residential Areas - Duplex, Terrace/Row houses, Apartment Blocks / Block of Flats / Real Estate, Gated Communities.

|  | Type | Plot Size in peri-urban area | Max. No. of household | Max. No. of Buildi-ng | Max. Plot coverage % | Max. Plot ratio | Max. No. of storeys | Minimum setbacks in metre |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | front | sides | rear |
| i | Duplex | 2001 – 4000m2 | 14 | 4 | 60 | 0.8 | 2 | 15 | 5 | 10 |
| ii | Terrace/Row houses | 4001– 8000m2 | 30 | 3 | 55 | 1.0 | 2 | 20 | 5 | 10 |
| iii | Apart-ment Blocks /Block of Flats /Real Estate | 8001–12000m2 | 50 | 5 | 50 | 2.5 | 5 | 20 | 5 | 10 |
| iv | Gated Comm-unities/ Conser-vation Reside-ntial areas | 12001-20,000m2 | 150 | 10 | 45 | 3 | 2 | 25 | 10 | 15 |

- Specific Standards for mixed use - Low Rise, High Rise and Skyscrapers.

|  | Type | Plot Size | Use | Max. No. of Building | Max. Plot coverage % | Max. Plot ratio | Max. No. of storeys | Setbacks in Metre |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | front | sides | rear |
| i | Lows Rise (1-5 Storey) | 2001 – 4000m2 | mixed use | 1 | 60 | 2.5 | 5 | 10 | 3 | 5 |
| ii | High Rise type 1. (6-10 Storeys) | 4001 – 8000m2 | mixed use | 2 | 55 | 5 | 10 | 15 | 3 | 5 |
| iii | High Rise type 2. (11-20 Storeys) | 8001 – 20,00m2 | mixed use | 3 | 50 | 8 | 20 | 15 | 3.5 | 5 |
| v | High Rise type 3. (21-35 Storeys) | 20,001 – 25,000m2 | mixed use | 4 | 45 | 12 | 35 | 20 | 5 | 7 |
| vi | Skyscra-pers (36-50 Storeys). | 25,001 – 30,000m2 | mixed use | 4 | 45 | 20 | 50 | 20 | 5 | 7 |
| vii | Above 50 storeys | Above 30,000m2 | Mixed use | Varia-ble | 40 | 10 | Varia-ble | 20 | 10 | 10 |

- Minimum Planning and Space Standards for Health Facilities

|  | Type | Plot Size | Max. No. of Buildings | Max.Plot coverage % | Max. Plot ratio | Max. No. of storeys | Setbacks in Metres |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | front | sides | rear |
| i | Dispen-sary/ clinic | 1000–5000m2 | - | 60 | 1.5 | 3 | 10 | 3 | 5 |
| ii | Health centre/ MCH | 0· 5–1ha | - | 55 | 2.2 | 4 | 15 | 3 | 5 |
| iii | Hospital | 2·5–10ha | - | 55 | 2.5 | 5 | 15 | 3.5 | 5 |
| v | District Hospital | 5 – 10ha | - | 45 | - | 5 | 20 | 7 | 10 |
| vi | Regional Hospital | 5 – 10ha | - | 45 | - | 5 | 20 | 5 | 7 |
| vii | Referral Hospital | 10 - 40ha | - | 40 | 2 | 5 | 25 | 7 | 7 |

- Minimum Planning and Space Standards for Education Facilities

|  | Type | Plot Size | Max. No. of Buildi-ngs | Max. Plot covera-ge % | Max. Plot ratio | Max. No. of storeys | Setbacks in Metres |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | front | sides | rear |
| i | Nursery School/Kindergarten day care Centre | 1200–1800m2 | 3 | 50 | 0.5 | 1 | 10 | 3 | 5 |
| ii | Primary School | 1·50–4·5 ha | - | 40 | 1.5 | 3 | 15 | 5 | 10 |
| iii | Secondary School | 2·5–5·0 ha | - | 40 | 2 | 5 | 15 | 5 | 10 |
| v | Polytechnic | 3·0–5·0 ha | - | 40 | 2 | 5 | 15 | 5 | 10 |
| vi | Education Centre | 1·2–2·5 ha | - | 50 | 2.5 | 5 | 20 | 5 | 7 |
| vii | Colleges /University College / | 5.0–10·0 ha | - | 50 | 5 | 10 | 20 | 10 | 10 |
| viii | University | 10·0–40·0 ha | - | 50 | 5 | 10 | 25 | 10 | 20 |

- Minimum Planning and Space Standards for Service Trade and Industries

|  | Type | Plot Size | Max. No. of Buildi-ngs | Max. Plot covera-ge % | Max. Plot ratio | Max. No. of storeys | Setbacks in Metres |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Front | Sides | Rear |
| i | Informal trade | 2,000–80,000m2 | - | 60 | 1.2 | 2 | 15 | 5 | 5 |
| ii | Small Scale /service Industries | 1,200 - 50,000m2 | - | 60 | 1.2 | 2 | 15 | 3 | 5 |
| iii | Medium/light/ heavy Industrial plot | 80,000m2 | - | 60 | 1.2 | 2 | 15 | 3 | 5 |
| v | Warehouse/ Godown | 80,000m2 | - | 50 | 1 | 2 | 15 | 5 | 7 |
| vi | Show rooms/yards | 40,000m2 | - | 60 | 3 | 5 | 15 | 5 | 7 |
| vii | Filling Station | 400 - 1,200m2 | 3 | 60 | 1 | 2 | 10 | 5 | 5 |
| viii | Petrol and Service Station | 2501 - 4000m2 | 3 | 50 | 1.5 | 2 | 10 | 5 | 5 |
| ix | Fire Station | 2000 - 3000m2 | 3 | 50 | 1.5 | 2 | 10 | 5 | 5 |

- Minimum distance between Petrol stations/Filling stations along the same side of roads shall be at least 200m.

- Minimum distance between Petrol stations/Filling stations on opposite side of a roads shall be the right of way of the

particular road.

- Minimum distance between Petrol stations/Filling stations in highly concentrated unplanned residential areas shall be at

least 500m.

- Plots for Petrol stations/Filling stations must be fenced with a strong concrete wall of a height of three meters.

The Technical Instruction No. 2 of 1991: Standard for Size, Location and Population per Petrol Station with Reference No. TP/01/197/60 dated 16th August, 1991 is hereby repealed.

- Minimum Planning and Space Standards for Public Facilities

- Minimum plot size for  Public Facilities

| Activity | Gross area/ person | Neighbourhood level | Community level (space) | District level |
| --- | --- | --- | --- | --- |
| Market | 0.4–0.5m2 | 1200–2500m2 | 0.5–1.5 ha. | 4.0 ha. |
| Shops/Shopping centres | 0.8–1.0m2 | 250–500m2 | 1.0–2.0 ha. | 8.0 ha. |
| Public areas/building | 0.25–0.5m2 | 800–2,500m2 | 0.32–1.5 ha. | 2.5 ha. |
| Religious sites | 0.3–0.4m2 | 1500 –2000m2 | 2000–4,000m2 | 0.8–1.6 ha. |
| Library | 0.15–0.2m2 | – | 1500–4,000m2 | 0.6–1.2 ha. |
| Community hall | 0.2–0.4m2 | 1000-3000 | 2000–4000m2 | 2.0 ha. |
| Post/Telecom. | 0.1m2 | – | 1000m2 | 1.0 ha. |
| Cemetery sites |  | 0.5–1.2 ha. | 2.0 ha. | 12.0 ha. |

- Plot coverage and plot ratio for Public Facilities

|  | Type | Plot Size Per Planning unit. | Max.Plot coverage % | Max. Plot ratio | Max. No. of storeys | Setbacks in Metres |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Front | Sides | Rear |
| i | Market | 1200 - 40,420m2. | 55 | 1.7 | 3 | 5 | 5 | 5 |
| ii | Public areas/building | 250 - 80,000m2 | 50 | 2.5 | 5 | 5 | 3 | 3 |
| iii | Religious sites | 1500 - 10,000m2 | 50 | 1.5 | 3 | 5 | 3 | 5 |
| v | Library | 1500 - 12,000m2 | 55 | 2.8 | 5 | 5 | 3 | 5 |
| vi | Community hall | 1000 - 20,000m2 | 50 | 1 | 2 | 5 | 5 | 3 |
| vii | Post/Telecom. College / | 1000 - 10,000m2 | 50 | 2.5 | 5 | 5 | 2 | 3 |

- Minimum Planning and Space Standards for Recreational Facilities

| Type | Planning Unit | Population/ Unit | Gross area/ Person | Plot Size |
| --- | --- | --- | --- | --- |
| Open spaces | Housing Cluster | 100–150 | 5.0–10.0m2 | 500–1,500 m2 |
| Neighbourhood Park | Neighbour-hood | 3,000–5,000 | 2.0–5.0m2 | 0.6–2.5 ha. |
| Community Recreational Park | Community | 10,000–20,000 | 1.5–2.5m2 | 1.5–4.0 ha. |
| Recreational Park (Amusement) | District/Town | 10,000–100,000 | 1-2m2 | 10.0–20.0 ha. |
| Central Park | Municipality/City | 100,000–1,000,000 | 1-2m2 | 20.0–100 ha. |

- Minimum Planning and Space Standards for Golf Course

| Course | Population/Unit | Parking | Average Length | Plot Size |
| --- | --- | --- | --- | --- |
| 9 holes | 5,000–25,000 | 100 cars | 750m | 50 ha. |
| 9 holes par 3 course | 5,000–25,000 | 100 cars | 750m | 50 ha. |
| 18 hole | 25,000–50,000 | 200 cars | 2170m | 50 ha. |

- Minimum Planning and Space Standards  for Passive Recreation

| Activity | Gross area/ 1000 persons | Neighbourhood level | Community level | District level |
| --- | --- | --- | --- | --- |
| Picnicking | 250–500m2 | 1.0–2.0 ha. | 5.0–10 ha. | 10ha-15ha. |
| Zoo, Arboretum | 0.5–1.0 ha. | 2.5–5.0 ha. | 10.0–15.0 ha. | 50ha- 100ha |
| Camping | 1.0–5.0 ha. | 10.0–25.0 ha. | 40.5–100.0 ha. | 100ha-150ha |

- Minimum Planning and Space Standards for Active Recreation

| Activity | Gross area/ 1000 persons | Neighbourhood level | Community level | District level |
| --- | --- | --- | --- | --- |
| Children play area | 0.2–0.4 ha. | 1.0–2.0 ha. | 4.0–8.0 ha. | 5 - 10.0 ha. |
| Play fields | 0.5–1.0 ha. | 2.5–5.0 ha. | 10.0–20.0 ha. | 10 - 50.0 ha |
| Sports fields and Stadia | 1.0–1.5 ha. | 5.0–8.0 ha. | 20.0–30.0 ha. | - 100.0 ha. |

- Minimum Planning and Space Standards for Public Facilities by Population Size

| Type | Cross area/person | Population size |  |
| --- | --- | --- | --- |
| Hotel site | 0.2m2 | 100,000 | 2.4 ha. |
| Commercial zone/Shopping mall | 0.3m2 | 100,000 | 3.6 ha. |
| Bar/Restaurant | 0.4m2 | 3,000+ | 3.6 ha. |
| Cinema hall | 0.3–10m2 | 400–800 seats | 4000–8000m2 |
| Lodge/Guest house |  |  | - 2 |

- Minimum Planning and Space Standards for Parking

12.1  Parking Requirements

| Land use | Parking Ratio |
| --- | --- |
| Housing estate | 2 per housing unit |
| Offices | 5/500m2 of total gross floor area |
| Commercial | 5/1000m2 of total gross floor area |
| Hospital | 10/50 beds |
| Hotel | 10/50 beds |
| Airport Hotel | 5/50 beds |

12.2 Special Parking Requirements

| SN | Level of provision | Area required (ha.) |
| --- | --- | --- |
| i | District Headquarters | 50.0 |
| ii | Regional Headquarters | 100.0 |
| iii | Commercial city | 300.0 |

12.3 Parking Lots

| Type of car | Angle of parking | area |
| --- | --- | --- |
| Buses and coaches | - | 40–50m2 |
| Car parking | - in line parking | 25–30m2 |
|  | - in 45 degrees | 20–30m2 |
|  | - in 90 degrees | 20–25m2 |
| Lorry parking | - in 90 degrees | 90–120m2 |
|  | - in 45 degrees | 135–145m2 |

- Minimum Planning and Space Standards for Electric Supply Substations

| SN | Type of electric supply lines in Kilowatts | Plot size (m) |
| --- | --- | --- |
|  | 33KV/11KV | 30 x 40 |
|  | 132KV/3KV | 50 x 80 |
|  | 220KV/132KV | 100 x 100 |
|  | 400KV/220KV | 200 x 200 |

13.1	Space Standards for Carriageways and Right of Ways

| Type | Right of Way (metre) | Carriageway (metre) |
| --- | --- | --- |
| Trunk road | 80 - 120 | 8 - 12 |
| Primary distributors | 60 - 80 | 6 – 8 |
| District distributors | 30 - 60 | 6 - 7 |
| Local distributors | 20 - 30 | 5 - 6 |
| Access Road in Residential area | 15.0–20.0 | 4 - 6 |
| Access Road in Industrial area | 15 - 20 | 4 - 6 |
| Access roads in shopping streets | 15 -20 | 10.0 |
| Cul-de-sac. | 10.0 | 5 |
| Rural roads | 20 –25 | 7.5 - 10 |
| Highways within urban areas | 45 | 6 - 7 |
| Pedestrian access | 5 | 2 |

13.2  Roads within Unplanned Settlements

| Type | Right of Way (metres) | Carriageway (metres) |
| --- | --- | --- |
| Primary Access | 12 – 15 | 5.0 – 7.5 |
| Secondary access | 10 – 12 | 5.0 - 6 |
| Tertiary access (one way) | 4 – 8 | 2 – 3 |
| Footpath | 2 – 4 | 2 |

13.3	Railways

| S/N | Type of railway truck | Right of Way in metres (ROW) | Distance from Centre Line in metres(CN) |
| --- | --- | --- | --- |
| i. | Trams and metro | 20.0 | 10.0 |
| ii. | Regional trains (Double truck line) | 50-60 | 25-30 |

- Planning and Space Standards for Public Utility Way leaves

14.1 Electricity Supply

| S/N | Type of electricity supply lines in kilowatts (KV) | Right of Way in metres (ROW) | Distance from Centre Line in metres (CW) |
| --- | --- | --- | --- |
| i. | 11 | 5.0 | 2.5 |
| ii. | 33 | 10.0 | 5.0 |
| iii. | 66 | 20.0 | 10.0 |
| iv. | 132 | 40.0 | 20.0 |
| v. | 220 | 60.0 | 30.0 |
| vi. | 400 | 60 | 30 |

14.2 Water Supply

| S/N | Type of water supply pipe | Right of Way in metres (ROW) | Distance from Centre Line in metres(CN) |
| --- | --- | --- | --- |
| i. | Trunk mains (main pipe) | 15.0 | 7.5 |
| ii. | Distributors | 2.0 | 1.0 |
| iii. | Fire Hydrants | 2 km. apart | - |

14.3 Gas/Oil supply

| S/N | Type of gas/oil pipe | Right of Way in metres (ROW) | Distance from Centre line in metres(CN) |
| --- | --- | --- | --- |
| i. | Main pipe | 60 | 30 |
| ii | Main Distributors | 30.0 | 15.0 |
| iii | Minor Distributor | 15 | 7.5 |
| iii. | Distributors | 2.0 | 1.0 |

- Planning and Space Standards for Airports

| S/N | Type of Airport | Area in Hectares |
| --- | --- | --- |
| i. | International Airport | 400.0 |
| ii. | Regional Airport | 150.0 |
| iii. | District Airport | 100.0 |
| iv. | Airstrip/landing ground | 60.0 |

- Planning and Space Standards for Railway Stations

| S/N | Type of Railway Station | Area in Hectares |
| --- | --- | --- |
| i. | Regional | 100.0 |
| ii. | District | 75.0 |
| iii. | Local | 50.0 |

- Planning and Space Standards for Solid and Liquid Waste Disposal

| S/N | Type of facility | Plot size (m2) |
| --- | --- | --- |
| i. | Sewerage treatment pond | 6500 |
| ii. | Landfill (Dumping) site | 40000 – 80000 |
| iii. | Collection point/centre | 400 |

- Planning Standards for Agricultural facilities

Show – grounds (open market)

| Level of Provision | Area Required (Ha.) |
| --- | --- |
| Village Centres | 25.00 |
| Minor Towns/Trading Centres | 50.00 |
| District Headquarters Towns . | 100.0 |
| Regional Headquarters Towns | 300.0 |
| Municipality | 300.0 |
| City | 400.0 |

- Minimum Planning and Space Standards for Agricultural lots/plots

| Level of Provision | Area required (m2) |
| --- | --- |
| Within urban areas | 4000 – 12,000 ( 1 acres – 3 acres) |
| In peri-urban areas (fringe zones) | 12001 + (3 Acres and above) |

- The Planning and Space Standards for plot size for communication pylons/communication towers is 20m x20 m.

- The Planning and Space Standards recommended minimum size for seasonal river, streams, pond, and swamp buffer zone of 15 - 30 metres from each side of the stream/ from High Water Mark and 60 metres from both sides of a river and from High Water Mark Lake, sea and ocean.

Dodoma,					                 William V. Lukuvi,

14th February, 2018		           Minister for Land, Housing and

Human Settlement Development